RLS- Architectural Design Document

System Overview

The RLS makes sure safe, reliable and controlled ignition of model rockets.

It consists of two communicating units:

- Launch Pad Unit : It is attached to the rocket motor, containing the battery, igniter connection, test/enable buttons and indicator lights.

- Control Unit: Operated remotely, containing ready/launch buttons and indicator lights, which sends commands to the LPU via a communication channel.

The goal is to enable safe ignition from a distance and provide feedback about circuit integrity and readiness before launch.

- Architecturally Important Aspects

Safety and reliability         : It prevents accidental ignition and ensures launch under safe

conditions.

Communication Integrity : CU and LPU must exchange reliable and authenticated signals.

Fault Detection                  : Detect broken wires, low battery or circuit faults

Real time responsiveness  : Acknowledge commands promptly.

Energy efficiency               : System should minimize energy draw when kept idle

Usability                             : LED indicators must give clear feedback.

2.  Proposed Solution

High-level Design

RLS follows a distributed embedded architecture with two microcontroller based subsystems communicating via a secure wireless link.

Launch Pad Unit

Hardware: Microcontroller, relay circuit to control igniter power, battery module, buttons and LED’s

Software:

- Idle mode: Disables all communication and circuit is open.

- Test mode: Applies low test current to igniter

- Enable mode: Enables communication channel with CU.

- Launch mode: On valid “launch” command, closes ignition relay momentarily.

Control Unit

Hardware: Microcontroller with same communication module, buttons and LEDs.

Software:

- Ready command: Sends “ready” signal to LPU

- Launch Command: Sends “launch” signal after receiving acknowledgement of “ready”

Communication Protocol

- Message based protocol with ack.

- READY_REQ  READY_ACK

- LAUNCH_REG  LAUNCH_ACK

- Timeouts and retransmission handle transient failures.

3.  Alternative Solutions

| Alternative | Description | Reason for rejection |
| --- | --- | --- |
| Simple wired circuit | Battery, switch, long wires to igniter | Accidental ignition, risk to user |
| Fully wired microcontroller communcation | Connect CU and LPU by a long shielded cable | Impractical for long distances (>50 m). Cables may break. |
| Wi Fi communication | Use Wi-Fi network and TCP/IP | Overkill for embedded context and energy cost. |

4.  Architectural patterns / Style chosen

- Layered Architecture – Separates hardware control from communication and logic layers, easing debugging and testing.

- Client – server pattern – The control unit acts as a client sending commands. The launch pad unit acts as a server responding to and executing requests.

- State machine architecture – Both units operate as finite state machines, making safety conditions easier to verify.

- Publish – subscribe – Internally used for event handling.

5. Lightweight Architectural Decision Record

See adr-001.md — “Choice of communication mechanism between Control Unit and Launch Pad Unit.

Conclusion

This architecture ensures

- Safety

- Reliability

- Usability

- Extensibility